For in-service engines, consider running cam

and lifterprelube in the oil all the time, not just

during break-in. Anotheralternative is to use

heavy-truck diesel-oil, which is formulated for1S

-wheelers and at present still has a full complement of traditionalantiwear

additives that have been significantly reduced in today'sstreet-Iegal

passenger car oils. (Though even diesel oils will startreducing zinc content in

2007 as big rigs gear up to receive catalyticconverters.) Comp Cams swears

by Shell Rotella T diesel oil for use inhigh-performance street cars. It's

available in both mineral-based andfull-synthetic formulations with both

types containing basically thesame superior additive package. Rotella

viscosities are generally higherthan today's modern formulations, but that's

not a detriment for classicmusclecars. Diesel oils also add a superior

detergent package tHat can keep the piston rings cleaner for better oil

consumption control. Thedrawback, if any, would be on a high-mileage

engine where blow-by cancause detergent to accumulate in the combustion

chamber, possiblycontributing to detonation.

Even better than diesel oil are

speciallyformulated racing motor oils. Although

the most expensive solution, these oils usually

contain even more antiwear additives than

dieseltruck oil, as well as other performanceenhancing

ingredientsspecifically designed for

hardcore, high-performance gasoline

engineusage. According to Cosworth's Thomas

Hayden, some diesel oils may nothave friction

modifiers that he claims are helpful in

preventing pistonscuff on high-performance

gasoline engines, especially if running

modern low multiviscosity oils. But Dan Arcy,

technical marketing manager forShell

Lubricants, takes issue with the importance of

friction modifiers,which he says "are only

present in the very low viscosity GF-4 oils

forfuel economy reasons."

Modern heavy-duty

truck diesel oils with

lots of ZDDPadditives

will bemarked "CI-4" or

"CI-4 Plus." They also

easily pass the API

Service SLgasoline

engine performance

test (but due to the

high ZDDP content,

notthe SL chemical

composition specs).

Such truck oils are an

economicaland effective

solution for flat-tappet

longevity, according to

manysources.

At any rate,

because they have

a full load of

antiwear

additives, today's

real racing motor

oils are sometimes

marked "for offhighway

useonly"

on the bottle. They definitely aren't embossed

with theconsumer-friendly starburst insignia.

Such racing oils won't meetmanufacturer's

warranty requirements for new vehicles, may

degradecatalytic converter performance in long-term use, and in some cases

havenot been formally submitted to the oil industry's current

benchmarkperformance test and validation procedure. But for older cars

runningflat tappets, they are the best oils available.

If you make provisionsto adequately lubricate the lifter/lobe interface, use

only qualitylifters, fill the sump with diesel or racing motor oils, and

followproper break-in procedure, any flat-tappet cam failures should

beminimized. It's a lot more effort than we've become accustomed to, butif

you still want to run a high- performance flat-tappet cam, it'ssomething

you'll just have to get used to doing.

- - ---

prODIefilS wlmnlgnperfo~

mance flat

tappets.

For in-service engines, consider running cam

and lifterprelube in the oil all the time, not just

during break-in. Anotheralternative is to use

heavy-truck diesel-oil, which is formulated for18

-wheelers and at present still has a full complement of traditionalantiwear

additives that have been significantly reduced in today'sstreet-Iegal

passenger car oils. (Though even diesel oils will startreducing zinc content in

2007 as big rigs gear up to receive catalyticconverters.) Comp Cams swears

by Shell Rotella T diesel oil for use inhigh-performance street cars. It's

available in both mineral-based andfull-synthetic formulations with both

types containing basically thesame superior additive package. Rotella

viscosities are generally higherthan today's modern formulations, but that's

not a detriment for classicmusclecars. Diesel oils also add a superior

detergent package tHat cankeep the piston rings cleaner for better oil

consumption control. Thedrawback, if any, would be on a high-mileage

engine where bl~~-by canca~se detergent to accumulate in the combustion

- chamoer, possiblycontributing to detonation. -- -

At any rate,

because they have

a full load of

antiwear

additives,today's

real racing motor

oils are sometimes

marked "for offhighway

useonly"

on the bottle. They definitely aren't embossed

with theconsumer-friendly starburst insignia.

Such racing oils won't meetmanufacturer's

warranty requirements for new vehicles, may

degradecatalytic converter performance in long-term use, and in some cases

havenot been formally submitted to the oil industry's current

benchmarkperformance test and validation procedure. But for older cars

runningfla~appets, th_ey are the best oils available.

If you make provisionsto adequately lubricate the lifter/lobe interface, use

only qualitylifters, fill the sump with diesel or racing motor oils, and

followproper break-in procedure, any flat-tappet cam failures should

beminimized. It's a lot more effort than we've become accustomed to, butif

you still want to run a high- performance flat-tappet cam, it'ssomething

you'll just have to get used to doing.